PHASE AND GAIN RESPONSE OF A CLOSED‑LOOP‑CONTROLLED HIGH VOLTAGE DC‑DC CONVERTER AT CRYOGENIC TEMPERATURES

Test Report

Scott Gerber

ZIN Technologies

Malik Elbuluk

The University of Akron

Richard Patterson

NASA Glenn Research Center 

Ahmad Hammoud

QSS Group, Inc.
NASA Glenn Research Center

Cleveland, Ohio

September 27, 2002
Phase And Gain Response Of A Closed‑Loop‑Controlled High Voltage 

DC‑DC Converter At Cryogenic Temperatures

Background
The phase and gain response of a 1 kW, 80-110V/500V closed-loop-controlled high voltage full-bridge dc-dc converter was evaluated at room temperature and at various temperatures down to ‑185 °C.  The converter design was based on that of a beam power supply for a space electric propulsion system [1], with the added capability of being able to operate over a very wide low temperature range.   The schematic of the full-bridge dc-dc converter is shown in Figure 1 and its control circuit is shown in Figure 2.  This cryogenic beam supply was previously tested in terms of efficiency, output voltage regulation, switching behavior and transient load testing [2,3].   Recent availability of a Venable Frequency Response System has made phase and gain response testing of the supply a reality.

Test Setup

Testing of the cryogenic beam supply was performed in a liquid nitrogen cooled chamber manufactured by SUN Systems.  A Venable Frequency Response System was used to measure phase and gain response over a frequency range of 10 Hz to 1 MHz.  Response measurements were recorded at chamber temperatures of 25 (C, ‑55 (C, ‑100 (C, -150 (C and –185 (C.  Due to limited availability of the frequency response system, initial testing was limited to a single operating point of 100V input voltage, 500V output voltage and a 600( load (half load ( 415W).  

Results and Discussion
The closed-loop controller as shown in Figure 2 was constructed from metal film resistors, solid tantalum, npo ceramic and mica capacitors, as well as, CMOS (timer, logic and operational amplifier) devices.  The controller utilized a Type I compensation network as defined in [4].  Previous testing utilized a 0.01μF compensation capacitor.  After first cut response measurements showed a phase margin of approximately 45º and a gain margin of 14 dB at 25(C, additional compensation was added by utilizing a 0.02μF compensation capacitor.  Response measurements for this configuration showed a phase margin of approximately 60º and a gain margin of 30 dB at 25 (C.  Table 1 lists the measured phase and gain margins of the controller at each test temperature.  As temperature was decreased to ‑100 °C both phase and gain margins were reduced to their minimum values of 45° and 16 dB, respectively.  As temperature was furthered decreased down to ‑185 °C the phase and gain margins returned to values close to those obtained at 25 °C. At all temperatures the controller maintains adequate phase and gain margins for controller stability.  

Table 1. Measured phase and gain margins of the controller at various test temperatures.

	Temperature (°C)
	Phase Margin (°)
	Gain Margin (dB)

	25
	60
	30

	-55
	60
	27

	-100
	45
	16

	-150
	48
	17

	-185
	53
	35


The phase and gain vs. frequency plots for the controller at test temperatures of 25 °C and ‑185 °C are shown in Figures 3 and 4, respectively.

Conclusion
A 1 kW 80-110V/550V closed-loop controlled full-bridge dc‑dc converter, designed to operate from 25 °C to ‑185 (C using commercially available components, was evaluated in terms of its frequency response over the same temperature range.  Testing was performed at a single nominal operating point of 100V input voltage, 500V output voltage and a 600( load (half load ( 415W).  Phase and gain response measurements were made over a frequency range of 10 Hz to 1 MHz.  The converter’s phase margin varied from a maximum of 60° at 25 °C to a minimum of 45° at ‑100 °C and the gain margin varied from a maximum of 35 dB at ‑185 °C to a minimum of 16 dB at ‑100 °C.   Measured phase and gain margins at ‑185 °C were similar to those measured at 25 °C.  At all temperatures the controller maintains adequate phase and gain margins for controller stability.  The results from this work indicate that control circuits can be designed and operated at very low temperatures well beyond normal operating temperatures.  

References

1.
Hamley, J.A., Pinero, L.R., Rawlin, V.K., Miller, J.R., Cartier, K.C., Bowers, G.E., “Development Status of the NSTAR Ion Propulsion System Power Processor,” 31st Joint Propulsion Conference and Exhibition, San Diego, CA, July 10‑12, 1995.

2.
S.S. Gerber, T. Miller, R.L. Patterson and A. Hammoud, “Performance of a Closed-Loop‑Controlled High Voltage DC-DC Converter at Cryogenic Temperatures,” IECEC, Colorado Springs, CO, 1998.

3.
S.S. Gerber, R.L. Patterson and B. Ray, “Operation of a High Voltage DC-DC Converter at Cryogenic Temperatures,” Proc. of the IASTED High Technology in the Power Industry, Oct. 27-30, pp. 463-470, Orlando, FL, 1997.

4.   H.D. Venable, “Optimum Feedback Amplifier Design for Control Systems,” Venable Industries, Inc.

Acknowledgments

This work was performed under the NASA Glenn Research Center, GESS Contract # NAS3-00145.
[image: image1.wmf]D

D1

0.01uF

1000V

D1

+

Vout

500V

-

A

-

A

IRFP254

A

IRFP254

At Room

Temperature

At Room

Temperature

330pF

1000V

470 ohm

8.0mH

current transformer 1000:1

8.0mH

0.01uF

1000V

+

B

330pF

1000V

0.68uF

400V

POWER TRANSFORMER

-

+

D1

D1

D1

D1

D1

Vin

80V/110V

470 ohm

0.01uF

1000V

100 ohm

0.68uF

400V

A

IRFP254

A

IRFP254

-

+

D1

D1

D1

D1

D1

C

470 ohm


Figure 1.  Full-Bridge DC-DC Cryogenic Beam Supply.

[image: image2.wmf]2.2K

15V

10K

15V

130pF

-

+

TLC2272

5

6

7

1N5817

CD4081

8

9

10

Current Transformer 1000:1

333K

1uF

CD4001

1

2

3

14

7

D

3.3K

10K

10K

1N5817

+8V

Current Feedback (Test Point)

Type I compensation

0.02uF

C comp

1N5817

Slope Compensation (Test Point)

10K

Rcomp

C4

CAP NP

CD4001

5

6

4

1K

10K

CD4081

5

6

4

CD4001

8

9

10

U15C

CD4071B

8

9

10

14

7

50 ohm

1K

CD4081

1

2

3

14

7

15V

10K

3.32K

B

Primary Current

10K

15V

14K

4.7K

15V

10K

30 ohm

C

U6B

CD4027

10

13

11

15

14

16

8

9

12

J

CLK

K

Q

Q

VDD

GND

S

R

Note: Power and ground bypassing not

shown.

10K

10uF

CD4027

10

13

11

15

14

16

8

9

12

J

CLK

K

Q

Q

VDD

GND

S

R

15V

750 ohm

12K

+ sense

15V

12K

15V

-8V

0.01uF

A

-

+

TLC2272

5

6

7

8

4

TLC555CP

3

4

8

1

5

2

6

7

OUT

RST

VCC

GND

CV

TRG

THR

DSCHG

10K

15V

CD4071

1

2

3

50 ohm

10K

CD4081

1

2

3

14

7

-

+

TLC2272

5

6

7

8

4

CD4027

6

3

5

1

2

7

4

J

CLK

K

Q

Q

S

R

10K

15V

1N5817

Output Voltage (500Vdc)

-

+

TLC354

1

6

7

0.01uF

- sense

At room temperature

External voltage reference

-

+

TLC2272

3

2

1

8

4

15V

3.21K


Figure 2.  Closed-loop controller.

[image: image3.png]=== — 1=
SE=E=== = 2
=
g
g
2| O
:
= z
= o
£

[
mﬁ 2oy
1084948684548

<
S
(4]

09~




Figure 3.  Phase and gain response vs. frequency of the controller at 25 °C.

[image: image4.png]60

-60

Ll

M

Gain

180

Phase

-180




Figure 4.  Phase and gain response vs. frequency of the controller at -185 °C.

1
NASA GRC 9/02


